RTES-6483 Project: “The Need for Speed”

| Course | RTES - 6483 |
| --- | --- |
| Course Instructor | Matthew Campisi |
| Student(s) | Rituraj Singh Gahlot (rsg8554) Aradhya Alamuru (aa9405) |
|  | Embedded Project |

Contents

1.  Gyroscope

  - Initializing Gyroscope

  - Configuring Gyroscope

  - Sampling rate and Calibration

  - Reading values from gyroscope

2.  LCD and Touch Screen

  - splashScreenView

  - configScreenView

  - mainScreenView

  - resultScreenView

3.  Speed and Distance Calculation

  - Circular Queue

  - Calculating average velocity

4.  Assumptions

5.  Results

1.	Gyroscope

  - Initializing Gyroscope

- Constructor:
Creating a gyroscope object int the main will call constructor to set the SPI frequency and format

Gyroscope::Gyroscope(SPI *spi_ptr, PinName ssel) { this->spi_ptr = spi_ptr; spi_ptr->frequency(10000000); spi_ptr->format(8,3); if(ssel == NC) { _spi_ssel_ptr = NULL; }else{ _spi_ssel_ptr = new DigitalOut(ssel, 1); } }

- Read and write register API’s

void Gyroscope::write_register(uint8_t reg, uint8_t val) { reg = reg & 0x7F; if (_spi_ssel_ptr != NULL) { _spi_ssel_ptr->write(0); } spi_ptr->write(reg); spi_ptr->write(val); if (_spi_ssel_ptr != NULL) { _spi_ssel_ptr->write(1); } } uint8_t Gyroscope::read_register(uint8_t reg) { uint8_t val; reg = reg ¦ 0x80; if (_spi_ssel_ptr != NULL) { _spi_ssel_ptr->write(0); } spi_ptr->write(reg); val = spi_ptr->write(0x00); if (_spi_ssel_ptr != NULL) { _spi_ssel_ptr->write(1); } return val; }

1.2	Configuring Gyroscope

- CTRL_REG1(20h): Value set to 0x0F
0x00  Bandwidth Selection 00

0x00  Data rate bit 00 to set output data rate to 95Hz

0x0F PD mode set to 1 : Normal mode
   	Z-axis enable to 1
 	Y-axis enable to 1
 	X-axis enable to 1

- CTRL_REG2(21h): Value set to 0x00
HPM1-HPM0  00 : Normal Mode
HPCF[3:0]  0000: Normal mode

- CTRL_REG3(22h): Value set to 0x00 (Disable interrupts)

- CTRL_REG4(23h): Value set to 0x00
FS1-FS0: Full scale selection set to 250dps
SIM: SPI serial interface mode selection mode to 4-wire

- CTRL_REG5(24h): Value set to 0x03
Out_Sel[1:0]: 11 : Out_Sel0 and Out_Sel1 enable
Hpen: 0: High-pass filter disable
FIFO_EN: 0: FIFO disable
BOOT: 0 : Normal mode

int Gyroscope::init() { front = -1, rear = -1; is_queue_full = 0; average_angular_velocity = 0; queue_count = 0; zero_count = 0; uint8_t id; for (int i = 0; i < 3; i++) { id = read_register(WHO_AM_I_ADDR); if (id == Gyroscope_ID) { break; } } if(id != Gyroscope_ID){ return -1; } update_register(CTRL_REG4_ADDR, 0x00, 0xC0); update_register(CTRL_REG5_ADDR, 0x03, 0x03); update_register(CTRL_REG3_ADDR, 0x00, 0x0F); //Disable interrupts update_register(CTRL_REG5_ADDR, 0x00, 0x40); //Disable FIFO update_register(FIFO_CTRL_REG_ADDR, 0x00, 0xE0); // configure FIFO bypass mode update_register(CTRL_REG4_ADDR, 0x00, 0x30); //Set Full Scale 250 update_register(CTRL_REG5_ADDR, 0, 0x10); //High Pass filter disable update_register(CTRL_REG2_ADDR, 0, 0x0F); //Disable HPF mode config update_register(CTRL_REG1_ADDR, 0, 0x30); //Bandwidth Selection 00 update_register(CTRL_REG1_ADDR, 0, 0xC0); //Output Data rate ODR :95hz i.e 0 update_register(CTRL_REG1_ADDR, 0x0F, 0x0F); //Enable gyroscope ; power up return MBED_SUCCESS; }

1.3	Sampling Rate and Calibration

- Sampling rate is set to 100Hz

- Sensitivity factor is set to 8.75 mdps/digit

- ZERO_RATE_NOISE set to 0.28, anything below this value is discarded for average velocity calculation

wait_us(10000); gyroscope.read_data_16(w_dps); gyroX = w_dps[0] * 0.00875f; gyroY = w_dps[1] * 0.00875f; gyroZ = w_dps[2] * 0.00875f; gyroX = gyroX + ZERO_RATE_NOISE; if (gyroX < 0.30) { gyroX = 0; }

1.4	Reading values from Gyroscope

- Read_data_16 will read 8-bit low value, auto increment the address and then reads the 8-bit high value

- 8-bit low value is left shifted and added with 8-bit High value to create 16-bit data each from x, y and z axis

void Gyroscope::read_data_16(int16_t data[3]) { uint8_t raw_data[6]; read_registers(OUT_X_L_ADDR, raw_data, 6); data[0] = (int16_t)(raw_data[1] << 8) + raw_data[0]; data[1] = (int16_t)(raw_data[3] << 8) + raw_data[2]; data[2] = (int16_t)(raw_data[5] << 8) + raw_data[4]; }

2.	LCD and Touch Screen

2.1	splashScreenView
	Main screen to display Project name with Start button

uint32_t splashScreenView(uint32_t screenWidth,uint32_t screenHeight) { uint32_t activeScreen; lcd.Clear(LCD_COLOR_WHITE); lcd.SetBackColor(LCD_COLOR_WHITE); wait(); lcd.SetTextColor(LCD_COLOR_RED); lcd.SetFont(&Font24); lcd.DisplayStringAt(0, LINE(3), (uint8_t *)"RTES", CENTER_MODE); lcd.SetTextColor(LCD_COLOR_MAGENTA); lcd.DisplayStringAt(0, LINE(4), (uint8_t *)"Need For Speed", CENTER_MODE); wait(); lcd.SetTextColor(LCD_COLOR_ORANGE); lcd.DrawLine(screenWidth / 4, 134, (screenWidth / 4) * 3, 134); lcd.SetFont(&Font16); lcd.SetTextColor(LCD_COLOR_BLUE); lcd.DisplayStringAt(0, LINE(9), (uint8_t *)"Rituraj Singh", CENTER_MODE); lcd.DisplayStringAt(0, LINE(10), (uint8_t *)"Aradhya Alamuru", CENTER_MODE); wait(); lcd.SetTextColor(LCD_COLOR_LIGHTBLUE); lcd.SetBackColor(LCD_COLOR_LIGHTBLUE); lcd.FillRect(screenMargin, screenHeight - 52, screenWidth - (2 * screenMargin), 44); lcd.SetTextColor(LCD_COLOR_WHITE); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, screenHeight - 36, (uint8_t *)"START", CENTER_MODE); while(1) { ts.GetState(&TS_State); touchX = TS_State.X; touchY = TS_State.Y; printf("x - %d, y - %d\n", touchX, touchY); if ((TS_State.TouchDetected) && (touchY < 40)) { activeScreen = CONFIG_SCREEN; break; } } return activeScreen; }

2.2	configScreenView

This screen will take input height from the user (Input height will be used in average velocity calculation)

uint32_t configScreenView(uint32_t screenWidth,uint32_t screenHeight) { uint32_t activeScreen; userHeight = 178; lcd.Clear(LCD_COLOR_WHITE); lcd.SetBackColor(LCD_COLOR_WHITE); wait(); lcd.SetTextColor(LCD_COLOR_BLACK); lcd.SetFont(&Font24); lcd.DisplayStringAt(0, 40, (uint8_t *)"Select Height", CENTER_MODE); lcd.SetFont(&Font16); lcd.SetTextColor(LCD_COLOR_GRAY); lcd.DisplayStringAt(0, 60, (uint8_t *)"(centimeters)", CENTER_MODE); lcd.SetFont(&Font12); lcd.DisplayStringAt(0, LINE(7), (uint8_t *)"We use your height to", CENTER_MODE); lcd.DisplayStringAt(0, LINE(8), (uint8_t *)"calculate speed", CENTER_MODE); lcd.SetTextColor(LCD_COLOR_DARKMAGENTA); lcd.SetBackColor(LCD_COLOR_DARKMAGENTA); lcd.FillRect(screenMargin, screenHeight - 52, screenWidth - (2 * screenMargin), 44); lcd.SetTextColor(LCD_COLOR_WHITE); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, screenHeight - 36, (uint8_t *)"GO!", CENTER_MODE); lcd.SetTextColor(LCD_COLOR_DARKRED); lcd.SetBackColor(LCD_COLOR_WHITE); lcd.SetFont(&Font24); while(1) { sprintf((char*)text1, " < %0.1f > ", userHeight); lcd.DisplayStringAt(0, 170, (uint8_t *)&text1, CENTER_MODE); ts.GetState(&TS_State); touchX = TS_State.X; touchY = TS_State.Y; printf("x - %d, y - %d\n", touchX, touchY); if (TS_State.TouchDetected) { if (touchY < 40) { activeScreen = MAIN_SCREEN; break; } if ((touchY > 80) && (touchY < 160)) { if (touchX > 160) { increaseHeight(); // wait(); } if (touchX < 80) { decreaseHeight(); // wait(); } } } } return activeScreen; }

2.3	mainScreenView

- This screen will display Time elapsed, Speed and distance travelled

- Ticker is used to display speed, time and distance after every 1 second

- Gyroscope’s x, y and z-axis angular velocities is read in this API

void mainScreenTickerFunc() { led != led; speed = avg_speed * ARC_length; dist += speed; lcd.SetFont(&Font24); // time elapsed; seconds sprintf((char*)text1, "%d", cnt); lcd.DisplayStringAt(0, 26, (uint8_t *)&text1, CENTER_MODE); // speed; m/s sprintf((char*)text2, " %0.2f ", speed * 3.6); lcd.DisplayStringAt(0, 124, (uint8_t *)&text2, CENTER_MODE); // distance; meters sprintf((char*)text3, "%0.2f", dist); lcd.DisplayStringAt(0, 224, (uint8_t *)&text3, CENTER_MODE); cnt++; } uint32_t mainScreenView(uint32_t screenWidth,uint32_t screenHeight) { uint32_t activeScreen; speed = 0; dist = 0; avg_speed = 0; cnt = 1; lcd.Clear(LCD_COLOR_WHITE); lcd.SetBackColor(LCD_COLOR_WHITE); lcd.SetTextColor(LCD_COLOR_RED); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, 6, (uint8_t *)"TIME", CENTER_MODE); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, 48, (uint8_t *)"seconds", CENTER_MODE); // lcd.SetTextColor(LCD_COLOR_DARKGREEN); // lcd.FillRect((screenWidth / 4) - 20, 80, (screenWidth / 2) + 40, (screenWidth / 4) + 20); lcd.SetTextColor(LCD_COLOR_RED); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, 100, (uint8_t *)"SPEED", CENTER_MODE); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, 148, (uint8_t *)"KM / H", CENTER_MODE); // lcd.SetTextColor(LCD_COLOR_DARKCYAN); // lcd.FillEllipse(screenWidth / 2, 200, screenWidth / 4, screenWidth / 8); lcd.SetTextColor(LCD_COLOR_RED); lcd.SetFont(&Font20); lcd.DisplayStringAt(0, 200, (uint8_t *)"DISTANCE", CENTER_MODE); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, 248, (uint8_t *)"METERS", CENTER_MODE); lcd.SetTextColor(LCD_COLOR_BLACK); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, screenHeight - 32, (uint8_t *)"Tap to End", CENTER_MODE); int err = gyroscope.init(); int16_t w_dps[3]; int count = 0; // button.fall(&Gyro_On); wait_us(1000000); ARC_length = userHeight/200; tick.attach(&mainScreenTickerFunc, 1.0); while(1) { wait_us(100000); gyroscope.read_data_16(w_dps); gyroX = w_dps[0] * 0.00875f; gyroY = w_dps[1] * 0.00875f; gyroZ = w_dps[2] * 0.00875f; gyroX = gyroX + ZERO_RATE_NOISE; if (gyroX < 0.30) { gyroX = 0; } gyroX1 = gyroX * 0.0174533f; avg_speed = gyroscope.average_Velocity(gyroX1); //Circular queue to calculate average velocity //float x1 = x * 0.0174533f;s //printf("%04d ¦ wx1: %f, wx: %f , temp: %d\n", count, x1,x, temp); printf("wx:%f wx1:%f temp:%f\n", gyroX, gyroX1, avg_speed); //printf("%04d ¦ wx: %+7.2f dps, wy: %+7.2f dps, wz: %+7.2f dps\n", count, x, y, z); //led = !led; count++; // touchscreen stuff ts.GetState(&TS_State); touchX = TS_State.X; touchY = TS_State.Y; printf("x - %d, y - %d\n", touchX, touchY); if ((TS_State.TouchDetected) && (touchY < 40)) { activeScreen = RESULT_SCREEN; break; } } return activeScreen; }

2.4	resultScreenView

- This screen will display total distance travelled and time for complete journey

- “TRY AGAIN” button will reset the LCD and gyroscope and will take you back to the first screen

uint32_t resultScreenView(uint32_t screenWidth,uint32_t screenHeight) { uint32_t activeScreen; lcd.Clear(LCD_COLOR_WHITE); lcd.SetBackColor(LCD_COLOR_WHITE); // wait(); lcd.SetTextColor(LCD_COLOR_DARKGREEN); lcd.SetFont(&Font24); lcd.DisplayStringAt(0, 40, (uint8_t *)"Done!", CENTER_MODE); lcd.SetTextColor(LCD_COLOR_ORANGE); lcd.DrawLine(screenWidth / 4, 72, (screenWidth / 4) * 3, 72); wait(); lcd.SetTextColor(LCD_COLOR_BLUE); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, 100, (uint8_t *)"Distance", CENTER_MODE); lcd.DisplayStringAt(0, 120, (uint8_t *)"Travelled", CENTER_MODE); wait(); lcd.SetFont(&Font24); lcd.SetTextColor(LCD_COLOR_RED); sprintf((char*)text1, "%0.2f meters", dist); lcd.DisplayStringAt(0, 137, (uint8_t *)&text1, CENTER_MODE); wait(); lcd.SetFont(&Font16); lcd.SetTextColor(LCD_COLOR_BROWN); lcd.DisplayStringAt(0, 190, (uint8_t *)"in", CENTER_MODE); lcd.SetTextColor(LCD_COLOR_MAGENTA); sprintf((char*)text2, "%d seconds", cnt); lcd.DisplayStringAt(0, 205, (uint8_t *)&text2, CENTER_MODE); wait(); lcd.SetTextColor(0xFF57068C); lcd.FillRect(screenMargin, screenHeight - 52, screenWidth - (2 * screenMargin), 44); lcd.SetBackColor(0xFF57068C); lcd.SetTextColor(LCD_COLOR_WHITE); lcd.SetFont(&Font16); lcd.DisplayStringAt(0, screenHeight - 36, (uint8_t *)"TRY AGAIN", CENTER_MODE); while(1) { ts.GetState(&TS_State); touchX = TS_State.X; touchY = TS_State.Y; printf("x - %d, y - %d\n", touchX, touchY); if ((TS_State.TouchDetected) && (touchY < 40)) { activeScreen = SPLASH_SCREEN; break; } } return activeScreen; }

3.	Speed and Distance Calculation

3.1	Circular Queue

- Circular queue of size 50 is implemented to capture 50 samples from gyroscope at run time at a rate of 100 samples per second

- Latest samples are En-queued and old samples are De-queued if queue is full

- A moving average is calculated at run time after each sample is read from the gyroscope. This moving average is used for calculating average velocity

void Gyroscope::push(float angular_x) { if(angular_x > 0){ if(queue_count >= size && is_queue_full == 0) { is_queue_full = 1; front = 0; } if(!is_queue_full) { rear = rear + 1; queue[rear] = angular_x; average_angular_velocity = ((average_angular_velocity * queue_count) + angular_x)/(queue_count+1); queue_count++; printf("1 : vel:%f qcnt:%d\n",average_angular_velocity,queue_count); } else{ average_angular_velocity = ((average_angular_velocity * queue_count) - queue[front])/(queue_count-1); rear = front; front = (front + 1) % size; queue[rear] = angular_x; average_angular_velocity = ((average_angular_velocity * (queue_count-1)) + angular_x)/(queue_count); printf("2 : vel:%f qcnt:%d\n",average_angular_velocity,queue_count); } } }

3.2	Calculating average velocity and Distance

- Gyroscope values are converted to radian/second using factor 0.0174533f

- This value is stored in circular queue and moving average is calculated after each sample is read from the gyroscope

- Moving average angular velocity is multiplied with leg length of the user to calculate linear velocity

- This linear velocity is used in ticker function with timer to calculate total speed and distance

void mainScreenTickerFunc() { led != led; speed = avg_speed * ARC_length; dist += speed; lcd.SetFont(&Font24); // time elapsed; seconds sprintf((char*)text1, "%d", cnt); lcd.DisplayStringAt(0, 26, (uint8_t *)&text1, CENTER_MODE); // speed; m/s sprintf((char*)text2, " %0.2f ", speed * 3.6); lcd.DisplayStringAt(0, 124, (uint8_t *)&text2, CENTER_MODE); // distance; meters sprintf((char*)text3, "%0.2f", dist); lcd.DisplayStringAt(0, 224, (uint8_t *)&text3, CENTER_MODE); cnt++; }

4.	Assumptions

- ZERO_RATE_NOISE is set to 0.2800, gyroscope reading below this is discarded

- Length of the leg is assumed to be half of the height of the user wearing the device

- If gyroscope reading is 0 for 30 consecutive samples, It is assumed user has stopped walking or running and moving average velocity is set to 0